In general, rotate the bottom knob to selects (highlight) things and top knob to changes values.
Press the bottom knob to enter a menu or perform the action highlighted. Press the top knob to go back or exit without changing something.

Changes to the Current Patch, Volume, FxSource, System Menu items or Calibration Menu items, will get written to non-volatile memory after a few seconds, so the next time you power on the synth it will be in the same state it was when you powered off.

Changing the Patch parameters values will stay active until you change the Current Patch, unless you make the change permanent by executing PatchPaste before changing the Current Patch.

Current Patch:  <patch number>
<patch name>

Volume:     0-100 in steps of 4

PatchPaste…
	Exit w/o pasting
	Destination: <destination patch numbe> change destination
	<destination’s current patch name> Press bot knob to edit destination name. Move cursor with bot knob. Change character with top knob. Must put spaces (between ‘/’ and ‘A’) at end to shorten a name. Press bot knob to keep new name or top knob to abort name change.
	PASTE writes patch to the destination with the destination’s current patch name

PatchSwap…
Same as PatchPaste, but the patch in destination gets moved to source location
(NOTE: PatchSwap is not working as of v.0.2.8)

Fx…
	Back
	FXSource:  Patch or Global (NOTE: can’t adjust global (v.0.2.8) except by “SaveGlobalFx”).
	SaveGlobalFx: 
		Exit w/o save
		Save saves current patch Fx to global Fx storage.
	DelayLevel: 0-127
	DelayTime: 0000-1270 (mS) in steps of 10
	DelayPong: 0=No pingpong (one delay, centered); 1-127 = 2 delays, 1 = center, 127 fully spread
	DelayFB: 0-127; 0 = no feedback, 127 almost 100% feedback
	DelayDamp: 0-127; 0=no LPF after delay; 127 max LPF after delay
	RvbLevel: 0-127
	RvbTime: 0000-5000 mS, in steps of 100mS
	RvbDensity: 0-127 (amount of reflections)
	RvbDamp: -10 - +10
	
	NOTE: there are issues with the Stereo chorus, right now 
	The Left channel is the ChrDry and Right is ChrsWet1+ChrsWet2 (this isn’t ideal but usable) 
	ChrsOn: Off, Mono(4k), Stereo (for centered stereo, Dry should be around 64 and dly1=dly2)
	ChrsDry: 0-127, amount of input signal (centered for stereo)
	ChrsLfoF: LFO frequency 0-12.7Hz in steps of 0.1
	ChrsFB: Feedback level, -64 - +63 (negative for inverted phase)
	ChrsDly1: 0-127 mS
	ChrsMod1: -64 - +63, amount modulation of ChrsDly1 (negative values for inverted phase)
	ChrsWet1: -64 - +63, amount of output from side 1, (negative values for inverted phase)
	ChrsDly2: 0-127 mS
	ChrsMod2: -64 - +63, amount modulation of ChrsDly2 (negative values for inverted phase)
	ChrsWet2: -64 - +63, amount of output from side 2, (negative values for inverted phase)

Osc1…

Osc2…

OscFilt1…

OscFilt2…

NoiseFilt1…

NoiseFilt2…

Noise...

Formant: Off,Woodwind, String (not implemented)

PatchAmp: 0-127

PatchCommon…
	Back
	Key Trig: Single, Multi (default single)
	BendRange: 0-12, (default 2)
	BendStep: rounds bend to nearest semitone (default off)

System Menu
	Aux In: 0-127, level of line in port
	Octave: global transpose Octave
	Semi: global transpose semitones
	Cents: global tuning in cents
	BreathCC: 1-5,7-11; default 2 (TODO: add Channel After Touch)

Calibration Menu:
	Back
	CompAttack: default 00.1
	CompRelease: default 020
	CompRatio: default 02.4
	CompThresh: default -12
	CompKnee: 024
	CompMUGain: 15
	NNBModCal: 60-100, default 87.  Sets what Note Number the filter’s Frequency 
		setting is true with no Breath Mod = 0;

Block diagram in progress…

Compressor: offers the following controls:
Attack rates 0.1 - 2000 mS, float
Release rates 0.1 - 2000mS, float
Ratio between 1 to 32767, float
Knee width between 0 to 40dB (soft to hard knee control), float
Threshold between 0 to -40dB, float
Makeup Gain between 0 to 40dB, float
Compresses the Input Signal based on either SideChain0 or SideChain1, uint8_t